IMPORTANT SAFEGUARDS

When using electrical equipment, basic safety precautions should always be followed including the following:

READ AND FOLLOW ALL SAFETY INSTRUCTIONS.

1. Do not use outdoors.

2. Do not mount near gas or electric heaters.

3. Use caution when servicing batteries.  Battery acid can cause burns to skin and eyes.  If acid is spilled on skin or in eyes, flush acid with fresh water and contact a physician immediately (If battery backup provided).

4. Equipment should be mounted in locations and at heights where it will not readily be subjected to tampering by unauthorized personnel.

5. The use of accessory equipment not recommended by the manufacturer may cause an unsafe condition.

6. Do not use this equipment for other than intended use.

7. All electrical connections must be in accordance with local codes.

8. Disconnect the power before starting the installation.

9. All servicing should be performed by qualified service personnel.

SAVE THESE INSTRUCTIONS

1. Product Overview

This product is certified by CSA/UL safety standards. This product has the advantages of beautiful appearance, convenient installation and maintenance, energy saving and consumption reduction, and long service life. Mainly used for airports, stations, docks, theaters, shopping malls, basements, confidential rooms, civil defense engineering and other places where personnel gather or move more, to provide safety exit instructions and emergency lighting in the event of power failure or power outage

2. Technical parameters

Voltage：AC120/347V 50-60Hz

Capacity：3.5W

Light source：8 bright LEDs

Charging time：≤24Hours

Emergency time：≥90minutes

Battery Specifications：4.8V500mAH(NI-MH)

3. Installation Method

This product provides a total of two installation methods: top mount and side mount. Please remove the fasteners, screws and plastic fasteners in the spare parts bag attached to the product and install them in Figure 1 and Figure 2 respectively.

Top Mount

Side Mount

4. Notice/Operation and Maintenance

The installation of lamps must be performed by qualified personnel only.

The installation of lamps shall be operated when the power supply is cut off.

If the product is connected to the AC power supply, the main charge indicator lamp shows on. The product operates with AC supply.

Depress test switch, product will remain light when switched to battery power. The indicator lamp shows off.

If the product is connected to AC supply, while the indicator lamp shows off, that may be some malfunction of the charging circuit. All Servicing should be performed by qualified personnel.

Suggest to allow battery to charge for 24 hours before first use.